Recurrent type of Road Safety Issues on PMGSY Roads

Safety Issue No 1: Start point

Description of Road Issue: Possible collision with through traffic on PMGSY.

Resulting Road Safety Risk: (5)

Recommendation to Address the Issue: Provide speed breaker before intersection to alert drivers. Provide advance informatory and Warning sign about the Road/Junction ahead

Safety Issue No 2: Curve on RHS

Description of Road Issue: Thick vegetation right at edge of road. Poor sight distance causing risk of head on collision.

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Clear vegetation on shoulder and ensure it is maintained clear in future. Provide curve warning signs.

Safety Issue No 3: Junction and Tree on RHS

Description of Road Issue: Possibility of accident at the junction after road is constructed, Risk of vehicles hitting the tree

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Provide Junction signs before the junction. Tree needs to be painted.

Safety Issue No 4: Tree on LHS and Curve

Description of Road Issue : Risk of vehicles hitting the tree, Thick vegetation right at edge of road.

Poor sight distance causing risk of head on collision.

Resulting Road Safety Risk: (2)

Recommendation to Address the Issue: Tree needs to be painted. Clear vegetation on shoulder and ensure it is maintained clear in future. Provide painted guard stones on outside shoulder to delineate the curve for both day and night driving. Provide curve warning signs.

Safety Issue No 5: Trees on both sides

Description of Road Issue: Risk of vehicles hitting the trees,

Resulting Road Safety Risk: (2)

Recommendation to Address the Issue: Tree needs to be painted.

Safety Issue No 6: Curve on LHS

Description of Road Issue: Thick vegetation right at edge of road. Poor sight distance causing risk of head on collision.

Resulting Road Safety Risk: (2)

Recommendation to Address the Issue: Clear vegetation on shoulder and ensure it is maintained clear in future. Provide curve warning signs.

Safety Issue No 7: “S” curve

Description of Road Issue: Thick vegetation right at edge of road. Poor sight distance causing risk of head on collision.

Resulting Road Safety Risk: (2)

Recommendation to Address the Issue: Clear vegetation on shoulder and ensure it is maintained clear in future. Provide curve warning signs.

Safety Issue No 8: Tree and Electric Pole on LHS

Description of Road Issue: Risk of vehicles hitting the tree and Electric Pole

Resulting Road Safety Risk: (2)

Recommendation to Address the Issue: Tree and Electric Pole needs to be painted

Safety Issue No 9: Edge Line and Center Line

Description of Safety Issue: Edge Line and Center Line are not provided.

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue:  Provide 100 mm thick Edge Line and Center Line.

Safety Issue No. 10: Cautionary Signboards missing.

Description of Safety Issue: Cautionary signboards are absent.

Resulting Road Safety Risk: (4)

Recommendation to Address the Issue:  Required cautionary signboards are to be provided as per IRC recommendations.

Safety Issue No. 11: Object Markers

Description of Safety Issue: Object markers which are used to indicate hazards and obstructions within the vehicle flow path are missing across the parapet walls of the Cross Drainage Works.

Resulting Road Safety Risk: (4)

Recommendation to Address the Issue:  The Cross Drainage Works needs to be properly marked with alternative white and black strips inclined at 45°.

Safety Issue No 12: Speed breaker

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Speed breaker needs to be painted in black and white strips and caution board need to be provided for road safety point of view.

Safety Issue No 13: Sign board

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Sign board has been provided but it is damaged needs to be provided proper sign board for road safety point of view.

Safety Issue No 13: Wheel guard and tree paint

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Trees on LHS and guard stone on RHS needs to be painted black white strip for road safety point of view.

Safety Issue No 14: Wheel guard paint

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Wheel guard on both side needs to be painted black and white strip for road safety point of view.

Safety Issue No 15: Guard stone provided

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue:  Nala on RHS needs to be provided guard stone for road safety point of view.

Safety Issue No 16: Personal Protection Equipments not found

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Work is in progress. Personal Protection Equipments need to be provided and labourers need to be encouraged to use personal protection equipments for their safety.

Safety Issue No 17: Unsafe merging with busy state highway

Description of Road Issue:  Road takes off from a busy state highway road with restricted visibility. But no safety measure has been considered at the merging.

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Provide speed breaker before intersection to alert drivers.

Provide advance informatory and Warning signs before approaching to the junction.

Safety Issue No 18: Curve without signage

Description of Road Issue: Guard posts have been provided but no signage observe in curve point

Resulting Road Safety Risk: (4)

Recommendation to Address the Issue: Provide signage with semi reflective as per specification for safe moving.

Safety Issue No 19: School and habitation without any safety measures

Description of Road Issue: No safety measures notice in these sensitive locations,

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Provide speed breaker with cautionary signage for road safety.

Safety Issue No 20:  Culvert without hazard marking and guard posts

Description of Road Issue: No safety measure found in culvert with high embankment

Resulting Road Safety Risk: (4)

Recommendation to Address the Issue: Wall should be painted with hazard markings. Provide guard posts as wheel guard to prevent errant vehicle from falling of road.

Safety Issue No 21:  Habitation and curve without any safety measures

Description of Road Issue: No safety precaution has been provided in this sensitive location.

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Provide anti honking restricted signage with speed breaker for safe moving.

Safety Issue No 22:  Under construction Culvert without any cautionary signage and diversion road

Description of Road Issue: Culvert construction is going on but no any cautionary signage or diversion road has been provided for safety.

Resulting Road Safety Risk: (4)

Recommendation to Address the Issue: Provide signage with retro reflective stickers for safe moving. Provide temporary traffic diversion road for safe moving of vehicle.

Safety Issue No 23:  Canal on RHS

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Canal on RHS along the road needs to be provided guard stone or protection wall provided for road safety point of view

Safety Issue No 24: Blind Curve

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Blind curve on RHS needs to be provided speed breaker and sign board for road safety point of view.

Safety Issue No 25: Check the height of Transmission Line

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Height of the ELC needs to be checked for vertical clearance (from the finished surface) and if required height of transmission line may be increased.

Safety Issue No 26: Curve with high embankment

Resulting Road Safety Risk: (4)

Recommendation to Address the Issue: Provide signage board in approach to the curve.

Provide guard post as wheel guard to prevent errant vehicle from falling off the road.

Safety Issue No 27: Habitation with unsafe hand pump

Description of Road Issue: Hand pump is very close to road alignment and it may cause traffic hazard for safe moving.

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Provide necessary steps for shifting of hand pump for safe living and moving of vehicle.

Safety Issue No 28: Habitation with restricted Right of Way and blind curves and junction

Description of Road Issue: Speed regulation is required at these sensitive locations to have safe journey on road and safe living for the residence

Resulting Road Safety Risk: (3)

Recommendation to Address the Issue: Provide speed breakers following proper guideline laid out in the DPR and IRC 99-1988 for up and down traffic at these locations. Provide road signs with speed restriction and cautionary information

Safety Issue No 29: Water body abutting road way

Description of Road Issue: Water body adjacent to road way. Vehicle restraint required to protect running vehicles

Resulting Road Safety Risk: (4)

Recommendation to Address the Issue: Provide guard post as wheel guard to prevent errant vehicle from falling off the road. Encourage plantation tree on the embankment to protect embankment as well as work as vehicle restraint system.

Resulting Road Safety Risk

Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high