(PART 1: ROAD INFORMATION)

| State: | Madhya Pradesh |
| --- | --- |
| District: | Sehore |
| Block: | Ichhavar |
| Road Number (Core Network): | NA |
| Construction Package Number or DPR reference number: | MP35-507 |
| Road Name: Length: | Dhamda-Golukhedi Road to Chhapari Taluk village 3.4 Km |
| RSA Stage: | DPR Final Design |

(PART 2: AUDIT INFORMATION)

| Date of Audit: | 23rd October 2011 |  |
| --- | --- | --- |
| Inspection Team and Participants |  |  |
| Name: | Role: | Organisation: |
| Mr. Jain | AM, PIU, Sehore I | MPRRDA |
| Subhash Nigam | Project Coordinator | PIC |
| A.K.Shrivastava | Road Safety Expert | TSC |
| Ms. Bhavna | Team Leader | PIC |
| Background to Inspection |  |  |
| Link road from Dhamda-Golukhedi Road to Chhapari Taluk village. Dhamda-Golukhedi Road originates from Bhopal-Dewas 4 lane road. The DPR of this road was already prepared. |  |  |
| Road Safety Audit (RSA) Process |  |  |
| Technical Support Consultants (TSC) reviewed the DPR provisions for road safety before visiting the road and initiating the RSA. The RSA was done by conducting transect walk on the proposed alignment. Date – 23rd October 2011 Weather – sunny Time – 2.00 PM to 4.00pm Traffic - LCVs, cars/vans, motor cycles, tractors, animal carts, cycles – Counted 25 numbers Road takes off from a habitation already linked by PMGSY road. |  |  |

(PART 3: ROAD SAFETY AUDIT FINDINGS)

| Safety Issue No 1 |  |
| --- | --- |
| Intersection with PWD Road as a Y junction. A school is located close to the road at the start point about 30m from the intersection. |  |
| Location |  |
| Chainage 0.00 to 0+030m at beginning point |  |
| Description of Road Issue |  |
| Possible collision with traffic on main road. School playground area abuts road shoulder without any fence/boundary wall. High risk of uncontrolled child running onto or across road with consequent vehicle/child collision. |  |
| Road Safety Risk |  |
| Exposure to Safety Issue: (5) Estimated as potential for traffic conflicts (e.g. braking, swerving, etc) caused by the issue Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Probability to Cause Accident: (3) Estimated as probability of traffic conflict resulting in an accident Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Consequence of Accident: (5) Estimated likelihood of personal injury or death caused by accident. Involvement of pedestrians/bicyclists versus vehicle would have severe consequence. If trucks are involved, the consequence would be even more severe. High speed of potential impact would have severe consequence.4 Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Resulting Road Safety Risk: (4.1) Combined rating equals sum of exposure rating plus probability rating plus consequence rating divided by 3. Higher the combined rating, greater the urgency of attending to the road safety issue. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Recommendation to Address the Issue |  |
| Junction to be properly designed, providing sufficient turning radius to traffic from Golukhedi Side. Provide speed breaker before intersection to alert drivers. Another speed breaker just before the school. Advance warning sign about the junction and school needs to be installed. |  |

| Safety Issue No 2 |  |
| --- | --- |
| Low level electric lines crossing the roads |  |
| Location |  |
| Chainage 250.00 metres. |  |
| Description of Road Issue |  |
| Risk of vehicles particularly loaded commercial vehicles getting entangled with electric line. |  |
| Road Safety Risk |  |
| Exposure to Safety Issue: (3) Estimated as potential for traffic conflicts (e.g. braking, swerving, etc) caused by the issue Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Probability to Cause Accident: (3 Estimated as probability of traffic conflict resulting in an accident Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Consequence of Accident: (3) Estimated likelihood of personal injury or death caused by accident. Involvement of pedestrians/bicyclists versus vehicle would have severe consequence. If trucks are involved, the consequence would be even more severe. High speed of potential impact would have severe consequence. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Resulting Road Safety Risk: (3) Combined rating equals sum of exposure rating plus probability rating plus consequence rating divided by 3. Higher the combined rating, greater the urgency of attending to the road safety issue. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Recommendation to Address the Issue |  |
| Electric lines need to be raised to safe level. |  |

| Safety Issue No 3 |  |
| --- | --- |
| Electric Pole close to the shoulder |  |
| Location |  |
| Chainage 1+000 metres |  |
| Description of Road Issue |  |
| Risk of vehicles hitting electric pole particularly at night. |  |
| Road Safety Risk |  |
| Exposure to Safety Issue: (2) Estimated as potential for traffic conflicts (e.g. braking, swerving, etc) caused by the issue Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Probability to Cause Accident: (2) Estimated as probability of traffic conflict resulting in an accident Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Consequence of Accident: (2) Estimated likelihood of personal injury or death caused by accident. Involvement of pedestrians/bicyclists versus vehicle would have severe consequence. If trucks are involved, the consequence would be even more severe. High speed of potential impact would have severe consequence. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Resulting Road Safety Risk: (2) Combined rating equals sum of exposure rating plus probability rating plus consequence rating divided by 3. Higher the combined rating, greater the urgency of attending to the road safety issue. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Recommendation to Address the Issue |  |
| Electric Pole needs to be shifted and painted for better visibility |  |

| Safety Issue No 4 |  |
| --- | --- |
| Tree close to shoulder |  |
| Location |  |
| Chainage 1800.00 metres |  |
| Description of Road Issue |  |
| Risk of vehicle hitting the trees particularly at night. |  |
| Road Safety Risk |  |
| Exposure to Safety Issue: (2) Estimated as potential for traffic conflicts (e.g. braking, swerving, etc) caused by the issue Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Probability to Cause Accident: (2) Estimated as probability of traffic conflict resulting in an accident Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Consequence of Accident: (3) Estimated likelihood of personal injury or death caused by accident. Involvement of pedestrians/bicyclists versus vehicle would have severe consequence. If trucks are involved, the consequence would be even more severe. High speed of potential impact would have severe consequence. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Resulting Road Safety Risk: (2.67) Combined rating equals sum of exposure rating plus probability rating plus consequence rating divided by 3. Higher the combined rating, greater the urgency of attending to the road safety issue. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Recommendation to Address the Issue |  |
| Tree needs to be painted so as to be clearly visible during night |  |

| Safety Issue No 5 |  |
| --- | --- |
| Sharp curve on the alignment. Well without wall close to road and on the curve. |  |
| Location |  |
| Chainage 2+100 meters to 2+400 metres. |  |
| Description of Road Issue |  |
| Well located close to sharp curve, high risk of vehicles falling into the well due to poor visibility. |  |
| Road Safety Risk |  |
| Exposure to Safety Issue: (4) Estimated as potential for traffic conflicts (e.g. braking, swerving, etc) caused by the issue Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Probability to Cause Accident: (4) Estimated as probability of traffic conflict resulting in an accident Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Consequence of Accident: (5) Estimated likelihood of personal injury or death caused by accident. Involvement of pedestrians/bicyclists versus vehicle would have severe consequence. If trucks are involved, the consequence would be even more severe. High speed of potential impact would have severe consequence. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Resulting Road Safety Risk: (4.13) Combined rating equals sum of exposure rating plus probability rating plus consequence rating divided by 3. Higher the combined rating, greater the urgency of attending to the road safety issue. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Recommendation to Address the Issue |  |
| Provide painted guard stones on outside shoulder to delineate the curve for both day and night driving throughout this section. Near the well put the guard stones in close interval. Provide curve warning signs |  |

| Safety Issue No 6 |  |  |
| --- | --- | --- |
| Narrow road on curve in the habitation area , low visibility because of the presence of structures, Presence of electric pole |  |  |
| Location |  |  |
| Chainage 3+100 m and 3+300 m |  |  |
| Description of Road Issue |  |  |
| Insufficient width for turning of vehicles particularly tractors and other commercial vehicles. Risk of accidents once road is constructed |  |  |
| Road Safety Risk |  |  |
| Exposure to Safety Issue: (3) Estimated as potential for traffic conflicts (e.g. braking, swerving, etc) caused by the issue Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |  |
| Probability to Cause Accident: (3) Estimated as probability of traffic conflict resulting in an accident Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |  |
| Consequence of Accident: (3) Estimated likelihood of personal injury or death caused by accident. Involvement of pedestrians/bicyclists versus vehicle would have severe consequence. If trucks are involved, the consequence would be even more severe. High speed of potential impact would have severe consequence. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |  |
| Resulting Road Safety Risk: (3) Combined rating equals sum of exposure rating plus probability rating plus consequence rating divided by 3. Higher the combined rating, greater the urgency of attending to the road safety issue. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |  |
| Recommendation to Address the Issue |  |  |
| Some portion of the structures especially the extensions without risking the main structure needs to be demolished for widening the road and making the curve smooth to the extent possible. This will also improve the sight distance at th e curve Speed breaker required at this place to slow down vehicular speed Electric Pole needs to be shifted |  |  |

| Safety Issue No 7 |  |
| --- | --- |
| School located close to the road alignment |  |
| Location |  |
| Chainage 3+400 m |  |
| Description of Road Issue |  |
| Risk of accident due to movement of small children |  |
| Road Safety Risk |  |
| Exposure to Safety Issue: (3) Estimated as potential for traffic conflicts (e.g. braking, swerving, etc) caused by the issue Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Probability to Cause Accident: (3) Estimated as probability of traffic conflict resulting in an accident Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Consequence of Accident: (3) Estimated likelihood of personal injury or death caused by accident. Involvement of pedestrians/bicyclists versus vehicle would have severe consequence. If trucks are involved, the consequence would be even more severe. High speed of potential impact would have severe consequence. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Resulting Road Safety Risk: (3) Combined rating equals sum of exposure rating plus probability rating plus consequence rating divided by 3. Higher the combined rating, greater the urgency of attending to the road safety issue. Scale: 1 - very low, 2- low, 3 – medium, 4 – high, 5 – very high |  |
| Recommendation to Address the Issue |  |
| Speed breakers and school warning signs are required |  |

(FORM A: EMPLOYER’s RESPONSE TO RSA FINDINGS)

| State: | Madhya Pradesh |
| --- | --- |
| District: | Sehore |
| Block: | Ichhavar |
| Road Number (Core Network): |  |
| Construction Package Number or DPR reference number: | DPR Final Design |
| Road Name: | Dhamda-Golukhedi Road to Chhapari Taluk village |
| Date of Audit: | 23rd October 2011 |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S No | PIC Details of issue | PIU Agree? Yes/No | PIU If disagree, explain why | PIC RSA suggestion(s) | PIU To be implemented? yes, no, partial (elaborate) | PIU If yes or partial: arrangements for implementation and timeline | PIU If no, describe an alternative action to be taken and arrangements for implementation |
| 1. | Intersection with PWD Road as a Y junction School very close to start point | Agree | {To be filled by PIU} | Junction to be properly designed, providing sufficient turning radius to traffic from Golukhedi Side. Provide speed breaker before intersection to alert drivers. Another speed breaker just before the school. Advance warning sign about the junction and school needs to be installed | Yes | Provisions made in DPR, Implementation of suggestions during construction. | {To be filled by PIU} |
| 2. | Low level electric lines crossing the roads, 250 m | Agree | {To be filled by PIU} | Electric lines need to be raised to safe level. | Yes | Provisions made in DPR, Implementation of suggestions during construction. | {To be filled by PIU} |
| 3. | Electric Pole close to the shoulder, 1000 m | Agree | {To be filled by PIU} | Poles need to be shifted/painted | Yes | Provisions made in DPR, Implementation of suggestions during construction. | {To be filled by PIU} |
| 4. | Tree close to shoulder, 1600m . | Agree | {To be filled by PIU} | Tree needs to be painted | Yes | Provisions made in DPR, Implementation of suggestions during construction. | {To be filled by PIU} |
| 5. | Alignment passes through Curves. Well without wall close to road and on the curve. Trees right on edge of the road, 2.1-2.5 km | Agree | {To be filled by PIU} | Provide painted guard stones on outside shoulder to delineate the curve for both day and night driving throughout this section Provide curve warning signs Construct concrete wall towards the well Trees need to be painted. Vegetation needs to be cleared | Yes | Provisions made in DPR, Implementation of suggestions during construction. | {To be filled by PIU} |
| 6. | Narrow road on curve, low visibility because of the presence of structures, Electric Pole, 3.1 km | Agree | {To be filled by PIU} | Some portion of the structures needs to be demolished for widening the road and making curve smooth. Speed breaker required at this place Electric Pole needs to be shifted | Yes | Provisions made in DPR (for Electric pole shifting) Implementation of suggestions during construction. | {To be filled by PIU} |
| 7. | School, end point | Agree | {To be filled by PIU} | Speed breakers and school warning signs are required | Yes | Provisions made in DPR, Implementation of suggestions during construction. | {To be filled by PIU} |