Madhya Pradesh Rural Road Development Authority

2nd - Wing, 5th Floor, Paryavas Bhawan, Bhopal

No. :       1079   /22/D-12/GM(T-1)/MPRRDA/11          Bhopal, Dated :   20/01/2011

addendum

The following amendments are made in the SSR issued by MPRRDA Dated    20-03-2009.

In chapter 3 – Earthwork, Erosion Control & Drainage would be added as below -

| Item No. | Description |  | Unit | Rate (Rs.) |
| --- | --- | --- | --- | --- |
| 3.2 | Construction of Embankment with Material Obtained from Roadway Cutting |  |  |  |
|  | D. | Deduct if Tractor Mount Grader, more than 65 HP (6 wheel - 4 wheel drive) is used in place of motor grader. | cum | 3.00 |
| 3.3 | Construction of Embankment subgrade & earthen shoulders with Material Obtained from Borrow Pits |  |  |  |
|  | E. | Deduct from item A or B if Tractor Mount Grader, more than 65 HP (6 wheel - 4 wheel drive) is used in place of motor grader. | cum | 3.00 |
| 3.9 | Construction of Embankment with Flyash/ Pond ash available from Coal or Lignite Burning Thermal Plants as Waste Material. Construction of embankment with flyash conforming to Table 1 of IRC:SP:58 obtained from coal or lignite burning thermal power stations as waste material, spread and compacted in layer of 200 mm thickness each at OMC by using motor grader & vibratory roller, all as specified in IRC:SP:58 and as per approved plans with lead upto 1000 m as per Technical Specification Clause 306. |  |  |  |
|  | C. | Deduct if Tractor Mount Grader, more than 65 HP (6 wheel - 4 wheel drive) is used in place of motor grader. | cum | 3.00 Count........2 |

// 2 //

In chapter 4 – Granular Sub-Bases, Bases (Non-Bituminous) and Shoulders would be added as below –

| 4.1 | Granular Sub-base with Well Graded Material (CBR 20 or more) (Table 400.1) Construction of granular sub-base by providing well graded material having CBR >= 20, spreading in uniform layers with motor grader on prepared surface, mixing by mix in place method with rotavator at OMC, and compacting with vibratory rollers of 80 to 100 kN static weight to achieve the desired density, complete as per Technical Specification Clause 401 for gradings mentioned in table 400.1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | E. | Deduct from item A or B if Tractor Mount Grader, more than 65 HP (6 wheel - 4 wheel drive) is used in place of motor grader. | cum | 12.00 |
| 4.2 | Lime Stabilisation for Improving Subgrade Providing, laying and spreading available soil in the subgrade on a prepared surface, pulverising, mixing the spread soil in place with rotavator with 2 per cent slaked lime having minimum 70 per cent of contents of CaO, grading with motor grader and compacting with the smooth wheel road roller at OMC to the desired density to form a layer of improved Sub-grade as per Technical Specification Cluase 403. |  |  |  |
|  | B. | Deduct from item if Tractor Mount Grader, more than 65 HP (6 wheel - 4 wheel drive) is used in place of motor grader. | cum | 12.00 |
| 4.3 | Lime Treated Soil for Sub-Base Providing, laying and spreading soil on a prepared sub-grade, pulverising, mixing the spread soil in place with rotavator with 4 per cent slaked lime with minimum content of 70 per cent of CaO, to achieve CBR value > 20, grading with motor grader and compacting with the road roller at OMC to achieve atleast 98 per cent of the max dry density to form a layer of sub-base as per Technical Specification Clause 403. |  |  | Count........3 |

// 3 //

|  | B. | Deduct from item if Tractor Mount Grader, more than 65 HP (6 wheel - 4 wheel drive) is used in place of motor grader. | cum | 12.00 |
| --- | --- | --- | --- | --- |
| 4.4 | Cement Treated Soil Sub-Base/Base. Providing, laying and spreading soil on a prepared sub-grade, pulverising, adding the designed quantity of cement to the spread soil, mixing in place with rotavator, grading with the motor grader and compacting with the road roller at OMC to achieve the desired unconfined compressive strength and to form a layer of sub-base/base as per Technical Specification Clause 404. |  |  |  |
|  | B. | Deduct from item if of Tractor Mount Grader, more than 65 HP (6 wheel - 4 wheel drive) is used in place of motor grader. | cum | 12.00 |

The addendum will be applicable with effect from 26.01.2011.

( M.K. Gupta )

Engineer-in-Chief

M.P. Rural Road Development Authority

Paryavas Bhavan, Bhopal

Endt. No. :              /22/D-12/GM(T-1)/MPRRDA/11      Bhopal, Dated :       /01/2011

Copy to :

- Director Project-I, NRRDA, 5th floor NBCC tower, Bhikaji Cama Place, New Delhi - 110066.

- Chief General Manager, Bhopal I & II, Indore and Jabalpur.

- General Manager (ALL), Project Implementation Unit ............................

Engineer-in-Chief

M.P. Rural Road Development Authority

Paryavas Bhavan, Bhopal